# 3D Printing Summary

| Metrics | Configuration #1 |
| --- | --- |
| Total Print Time (min) | 61 |
| Total Number of Components | 3 |
| Typical Total Mass (g) | 8.65 |
| Typical Number of Print Setups | 1 |

# 3D Printing Settings

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rocket-Switch-Interface-Bottom-Case.stl | 1 | 0:29 | 4.48 | 15 | N | 0.2/0.4 | - It doesn’t require support - Orient upright so that the screw hole and reset hole are on the print bed. |
| Rocket-Switch-Interface-Top-Case.stl | 1 | 0:26 | 3.64 | 15 | N | 0.2/0.4 | - It doesn’t require support - Orient upright so that the text is on the print bed. |
| Rocket-Switch-Interface-Jig.stl | 1 | 0:06 | 0.53 | 15 | N | 0.2/0.4 | - It doesn’t require support - Orient upright so that the rectangular cut is on the print bed. |

# Post-Processing

- Remove any burrs or sharp edges if needed.

# Examples of Quality Prints

Rocket-Switch-Interface-Bottom-Case

Rocket-Switch-Interface-Top-Case

Rocket-Switch-Interface-Jig